One strategy I've used throughout the design if this driver is to make small sacrifices in accuracy for the sake of space and speed. The audio generated may not be 100% mathematically accurate, but the differences aren't typically perceivable for the average human. While this might not be acceptable for something like a professional recording studio, it's perfectly fine for many other uses, such as games.

This sound driver plays and mixes multiple channels of tones, noise and PCM samples (PCM samples must be 11KHz 8-bit signed). Frequency (except on PCM sounds), volume, and volume envelopes can be specified and adjusted for each channel. Internally, the driver works on 16-bit samples, and comes in two versions: 11KHz with up to 9 channels and 22KHz with up to 6 channels.

The sound driver works in a way similar to Parallax's graphics and input drivers. It is made up of two parts: a SPIN-based API that can be called from another object, and the actual assembly driver, which runs on its own separate COG.  For a program to use this driver, it calls any of the SPIN API functions, which in turn, pass the commands along to the COG running the actual assembly.

Explain the sound clock and "sound clock ticks".

| Sound clock | Action (for 6-channel 2KHz version) |
| --- | --- |
| 0 | Poll variables for channel 0, compute all channels, mix and output. |
| 1 | Poll variables for channel 1, compute all channels, mix and output. |
| 2 | Poll variables for channel 2, compute all channels, mix and output. |
| 3 | Poll variables for channel 3, compute all channels, mix and output. |
| 4 | Poll variables for channel 4, compute all channels, mix and output. |
| 5 | Poll variables for channel 5, compute all channels, mix and output. |
| 6 | Compute all channels, mix and output. |
| 7 | Compute all channels, mix and output. |
| 8 | Poll variables for channel 0, compute all channels, mix and output. |
| 9 | Poll variables for channel 1, compute all channels, mix and output. |
| ... | ... |

Overview:

This driver generates and outputs 11KHz or 22KHz audio one sample at a time using a "pseudo PWM" technique. Section 9 of the Hydra Hardware Manual describes this technique in detail, but the important thing is this: with a little bit of circuitry already on the Hydra, the COG's counter can be set up so we can simply write 32-bit samples to the FRQx register at 11KHz/22KHz to generate sound. Note however, that although the final sample being output is 32-bit, this driver operates on 16-bit samples internally before converting the final output sample to 32-bit.

The driver can play up to 6 (22KHz) or 9 (11KHz) sounds simultaneously. When you play a sound, there are a few different values used to describe the sound:

| Communication Variables: audio_pin (Sent as PAR) Communication variables for channel 0 Communication variables for channel 1 ... Communication variables for channel (n-1) | Each set of communication variables: pending_shape pending_freq pending_duration pending_volume pending_amp_env pending_pcm_start pending_pcm_end |
| --- | --- |

Communication Protocol between SPIN and ASM

The communication is done through the pending_* variables.

The pending_* variables are normally set to NOTHING_PENDING ($0). To change

a setting, write the desired new values to the appropriate pending_*

variables. Then set bit 31 of pending_shape* to 1. The ASM driver polls the

pending_shape* variables. When it sees a 1 in bit 31, it will poll the rest

of the pending_* variables for that particular channel. When it encounters

a value other than NOTHING_PENDING in any of those variables (including

bits 30..0 of pending_shape*), it will activate the new setting, clear bit 31

of pending_shape*, and write NOTHING_PENDING back to the variable to signal

it has received the new value.

| Name | Description | Possible Values |
| --- | --- | --- |
| Channel | The channel on which to play the sound. | 0-5 (22KHz), 0-8 (11KHz) |
| Shape | The type of waveform, such as sine wave, noise, or PCM | Normal Sounds: SHAPE_SINE SHAPE_SAWTOOTH SHAPE_SQUARE SHAPE_TRIANGLE SHAPE_NOISE SHAPE_PCM_8BIT_11KHZ Special-Purpose Values: SHAPE_IGNORE SHAPE_SILENT SHAPE_RELEASE |
| Frequency | The desired sound frequncy. Ignored if the sound is a PCM. | 8-Nyquist No: 0: NOTHING_PENDING 1-7: Problem with envelopes >Nyquist: (Nyquist–Shannon sampling theorem http://en.wikipedia.org/wiki/Nyquist-Shannon_sampling_theorem) Or, use NOTE_* constants |
| Duration | Length of time to play the sound, measured in 11KHz or 22KHz "ticks". Infinite duration is possible. | Bits 0-30: SAMPLE_RATE*number of seconds. Even if the duration is infinite, this is still used for the envelope (explained later). Bit 31: 0 - Finite duration 1 - Infinite Duration |
| Volume | The desired volume. | 1 (Min Volume) - 255 (Max Volume) |
| Envelope | An envelope to be applied to the volume. The volume will change as a function of time. This can be used to simulate Attack-Decay-Sustain-Release. | Eight 4-bit nybbles from $0 (0% of the specified Volume, no sound) to $F (100% of the specified Volume, full volume), to be applied least significant nybble first and most significant nybble last. See "Explanation of Envelopes and Duration" for details. |
| PCM Start Address | The address in HUB memory where a buffer of sound samples begins. | Any address in HUB memory. |
| PCM End Address | The address where the buffer of sound samples ends. | Any address in HUB memory (must be greater than the starting address) |

Freq:

Any 32-bit value except 0 will be accepted, but there may be precision errors at extremely high and low values. But any human-perceptible frequency should work fine (

Explain pending_*_* vars (and audio_pin).

Within the driver, each channel has the following values:

| Name | Description | Values |
| --- | --- | --- |
| theta | The current position in the wave. (Not used for PCM) | $0000 (0 degrees) - $2000 (360 degrees) |
| theta_delta | The value by which to advance theta every sound clock tick. | Calculated by the formula: ($2000 * Frequency) / SAMPLE_RATE |
| theta_cycled | Whether or not the waveform has just completed a full cycle. Needed for generating noise. | If a cycle has just been completed then 1, otherwise 0. |
| shape | The type of waveform, such as sine wave, noise, or PCM. | The SHAPE_* value minus one. Ie, if the shape is a sine wave, then the value is SHAPE_SINE-1. The reason for this off-by-one is explained below. |
| stop_time | When the sound clock reaches this value, the envelope will advance to the next envelope segment (or the sound will stop if it's reached the end of the envelope). (See "Explanation of Envelopes and Duration" for an explanation of envelopes.) | The value of the sound clock when the currently envelope segment started plus env_seg_duration. If the duration is flagged as infinite and , then stop_time is 0 (because sound_clock is set to skip 0). (See "Explanation of Envelopes and Duration" for an explanation of envelopes.) |
| volume | The sound's volume. | The same 8-bit value passed to the driver as the volume. |
| amp_env | The envelope used to modify sound's the amplitude (volume). (See "Explanation of Envelopes and Duration" for an explanation of envelopes.) | Starts out as the same 32-bit value passed to the driver as the envelope. But, as each nibble is used, it gets shifted out to the right. When this value reaches 0, the sound stops. (See "Explanation of Envelopes and Duration" for an explanation of envelopes.) |
| env_seg_duration | The duration of each envelope segment. (See "Explanation of Envelopes and Duration" for an explanation of envelopes.) | One eighth of the sound's duration. (See "Explanation of Envelopes and Duration" for an explanation of envelopes.) |
| lfsr | Linear-Feedback Shift Register: Used for pseudo-random number generation to use as noise. (For an explanation of LFSR's, see http://en.wikipedia.org/wiki/LFSR) | The current pseudo-random number. |
| pcm_start | The address in HUB memory where a buffer of sound samples begins. This isn't strictly necessary at the moment, only pcm_curr and pcm_end are needed, but I've left it in to facilitate adding automatically looping sounds at a later date. | The same address passed to the driver as the PCM Start Address. |
| pcm_end | The address where the buffer of sound samples ends. | The same address passed to the driver as the PCM End Address. |
| pcm_curr | The address of the current sample in the PCM buffer. Also stores a 1-bit counter in the 22KHz version. | A HUB address between pcm_start and pcm_end. In the 22KHz version, bit 31 is used as a 1-bit counter to help stretch the 11KHz samples into 22KHz, so they don't play too fast (Remember, HUB addresses only use 16 bits). |

As you can see, some of the values are taken straight from the commands sent to the driver, some are slightly modified, and some are calculated.

The driver does some brief initialization and then goes into a main loop.

Process Overview:(diagram)

1. Initialization

2. Process requests from API

3. Generate sample

4. Go back to step 2

Initialization:

The initialization phase is fairly simple and straightforward. It's only nine instructions and resembles the beginning of any typical Propeller ASM routine: It grabs a parameter through PAR and sets up some registers and an output pin.

The parameter sent through PAR is the pin to use as audio output. On the Hydra, this is always 7. If you're designing a custom Propeller circuit, you should be able get stereo or surround sound: Just connect up to 7 different Propeller I/O pins to separate audio outputs, and run a copy of this driver on a separate COG for each audio output pin (Note that I haven't actually tested this).

After setting up the audio pin, the initialization section sets up a clock value for a WAITCNT in step 4 to wait for (to synchronize the main loop to either 22KHz or 11KHz). Then, counter A is set up for "pseudo PWM" by using Single Duty Cycle mode to feed the audio output pin.

Process requests from API:

For one channel (determined by low two/three bits of sound_clock): (diagram) Explain round-robin updating of channel data (maybe compare to HUB's round-robin servicing of COGs), and why it's not necessary to check all the channels every clock. Explain tradeoff: If updated all channels every sound clock, would only need one set of  pending_* vars in the HUB ram (but HUB is not as memory-limited as the COGs anyway). At some point, suggest two channel at a time hybrid approach, if needed.

For current channel, get each pending_* into _pending_*. Then, for each of them, check if a change in state is pending. If so, do needed parameter->channel data processing.

Generate Sample:

The heart of the driver. Actually generates and outputs the sound. Remember, because of Andre's tip for the COG counter, just need to write the final, mixed sample to FRQA.

So, general process is this: For each channel, generate raw sample from chosen shape based on theta (diagram) (also explain dispatch, jump table), then update and apply envelope (includes stopping), then add all together (diagram), write to FRQA.

- Initialization

- Main sound clock loop (Runs at 11/22KHz, each iteration is one "sound clock")

- Process requests for one of the channels

- Check if a request is pending (by checking bit 31 of pending_shape)

- If so, grab each communication variable (into _pending_*), and signal that they have been received (by resetting them to NOTHING_PENDING)

- Convert each into channel data and store

- Loop to samples from each channel

1. Retreive state data

2. Jump table to get raw sample

3. Advance theta

4. Store updated state data

5. Update and apply envelope (envelope applied to volume, not sample)

(needs deeper walkthrough) (diagram?)

a.

b...

6. Apply volume

7. Add into mixer

8. If any more channels, point to next set of channel data and repeat from 1

9. Crank volume high as possible, signed->unsigned addition (ie 0 -> long_half), output to FRQA

10. Reset pointers back to first channel's data, wait to sync to 11/22KHz (diagram), and go back to process API requests.

(diagram)

1. Retreive state data

2. Jump table to get raw sample

3. Advance theta

4. Store updated state data

5. Update and apply envelope (envelope applied to volume, not sample)

(needs deeper walkthrough) (diagram?)

a.

b...

6. Apply volume

7. Add into mixer

8. If any more channels, point to next set of channel data and repeat from 1

9. Crank volume high as possible, signed->unsigned addition (ie 0 -> long_half), output to FRQA

10. Reset pointers back to first channel's data, wait to sync to 11/22KHz (diagram), and go back to process API requests.

Explain custom multiplies to improve speed.

Explanation of Envelopes and Duration:

If you like, you can perform your own volume envelope by passing an envelope of $FFFF_FFFF to the driver, and then adjusting the volume just like you would adjust the frequency in a frequency sweep. But the driver has envelope functionality of it's own built in.

You can implement your own envelope function, perhaps using a smooth interpolation, by passing an envelope of $FFFF_FFFF to the driver, and then periodically sending new values for volume to the driver. Essentially, this is just like performing a frequency sweep, but adjusting the volume instead.

There are a number of size and speed optimizations that could be done. The theta_cycled, volume and pcm_curr fields of the channel data could likely be combined, for example. Also, it might be possible to eliminate the 1-bit counter tacked onto pcm_curr in the 22KHz version by just reading bit 0 of sound_clock. And I think there may be a place or two where a couple instructions could be replaced with a CMPSUB. But in any of these, you would still have to be careful to balance costs versus the savings. Combining volume and pcm_curr, for example, may require extra instructions to merge and separate the values as needed. It's all a balancing act.