ZOG: GNU C/C++ for the Propeller

January 28, 2011

Manual by:

David Betz

Michael Rychlik

Software by:

Michael Rychlik

David Betz

Steve Denson

ZOG: GNU C/C++ for the Propeller	1

1	Introduction	4

1.1	What is Zog?

1.2	Supported Boards

1.3	Installing ZOG

1.4	Installing the Serial Download Helper

1.5	hello

1.6	filetest

1.7	xbasic

2	Building Programs	9

2.1	Board Configurations

2.2	Make Files

3	ZOGLOAD	11

3.1	Description

3.2	Selecting the Port (-p)

3.3	Selecting the Board Configuration (-b)

3.4	Writing a File to Flash or RAM (-wm)

3.5	Writing a File to the SD Card (-wf)

3.6	Load the helper program to Propeller RAM (-h)

3.7	Setting the Program Name (-n)

3.8	Writing a Bootloader to the EEPROM (-we)

3.9	Setting the File to be Loaded by the Bootloader (-wb)

3.10	Running a Program (-r)

3.11	Terminal Emulation (-t)

4	C Runtime Library	15

4.1	Initialization

4.2	Serial I/O

4.3	Keyboard Input

4.4	TV Output

4.5	File I/O

5	Drivers	17

5.1	FdSerial

5.2	TvText

5.3	Tv2Text

5.4	Keyboard

6	Propeller Interface	22

6.1	Hub Memory Variables

6.2	Memory Map

6.3	Propeller Registers

6.4	Propeller Functions

# Introduction

## What is Zog?

Zog enables running C and C++ programs compiled with GCC, the GNU Compiler Collection, on the Propeller multi-core micro-controller from Parallax Inc.

There is no Propeller architecture target for GCC. To overcome this Zog provides a virtual machine implementing the instruction set of a CPU architecture for which there is a GCC target, namely the Zylin CPU (ZPU) from Zylin Consulting. The ZPU architecture provides a 32-bit processor with a very minimal instruction set, each ZPU instruction is a single byte, it is therefore a perfect candidate for emulation on the Propeller as the entire bytecode interpreter can be implemented in PASM within a single COG. Hence the name ZOG, ZPU in a Cog. Further, as each instruction is a single byte the ZPU is a good match for Propeller systems having external RAM or ROM that is connected over an 8 bit data bus.

Zog can be used to run compiled C, C++ binaries on a bare Propeller where the binaries are located within the Propellers HUB RAM space. This limits the binary size to a little less than 32KBytes. Fortunately there are Propeller systems available that provide memory external to the Propeller chip and Zog can also make use of such memory expansion from which it can execute much larger binaries and deal with much larger data sets. Up to 32MBytes currently!

This document describes how get C and C++ programs running on some of those Propeller platforms with the Zog virtual machine.

If you are interested in the details of the ZPU architecture and instruction set you will find it on the Zylin Consultants WEB site:

http://opensource.zylin.com/zpu.htm

## Supported Boards

This version of ZOG supports the following boards:

- Parallax Propeller C3 board

- Gadget Gangster Propeller Platform board with the SDRAM module. To use TV output and PS2 keyboard input requires the SDRAM/TV module.

## Installing ZOG

This release only runs under Windows. Since you are reading this file you must have unzipped the zog-cygwin.zip file successfully. This should have created a directory called zog-cygwin. Open up a Windows Command prompt and cd to the zog-cygwin directory.

Then, edit the setenv.bat file in this directory so that the ZOGPATH environment variable points to the directory where this README.txt file is located and set the PORT variable to the COM port that talks to your Propeller board. The ZOGPATH variable must be set using the Cygwin path syntax.

In other words, if you've unzipped the zog-cygwin.zip file in the directory

C:\Users\myname

then you should set the ZOGPATH environment variable like this:

set ZOGPATH=/cygdrive/c/Users/myname/zog-cygwin

In other words,

C:\

becomes

/cygdrive/c/

and the rest of the path stays the same with the exception of converting all backslashes to forward slashes. If you've installed on a different drive the procedure is similar.

For instance,

D:\

becomes

/cygdrive/d/

You should also change the BOARD environment variable to reflect the Propeller board you will be using. Select c3 for the Parallax Propeller C3 board and sdram for the Gadget Gangster Propeller Platform with SDRAM board. There is an additional c3ram board type that you can use if you want to run code from the C3 SPI SRAM rather than the C3 SPI flash.

Once you’ve completed editing your setenv.bat file type setenv at the Windows command prompt to run the setenv batch file.

This should setup your environment to work with your Propeller board.

## Installing the Serial Download Helper

In order to download a ZOG image to either Propeller external memory or to the SD card zogload needs a helper program running on the Propeller. Normally you should write the helper program to EEPROM before downloading any ZOG images. You can do this with the following command:

zogload –we helper

This only needs to be done once. After that, each time zogload wants to start a download it will reset the Propeller chip and the helper will run to assist in the download. If the Propeller is reset by some other means like power up or the reset button, the helper program will timeout after one second and will look for an SD card file named zogload.run  and will run the ZOG image whose name is in that file.

If you don’t want to write the helper program to EEPROM you can specify the –h option on the zogload command line before the file to download and zogload will download the helper program before downloading the ZOG image. This allows you to use zogload without disturbing what is programmed into your EEPROM but is also slower since it has to download the helper each time.

Sample Programs

You can get a feel for writing ZOG C programs by looking at the examples in the tests directory.

The hello test program requires a TV and PS2 keyboard to be connected to your Propeller board. The filetest, setjmp, and xbasic test programs only require the USB serial connection that you use to download programs to your board.

## hello

To build the hello test program, cd to the tests\hello directory and type make. This should run the ZPU C compiler to compile and link the hello test program. You will find the resulting program in tests\hello\bin\hello.zbn. Files with the extension .zbn are ZOG binary images that are ready to load into your Propeller board. To load the hello test program, type make run. This should load the hello test program into your board and start it running. You should see semi-interesting stuff on the TV you have plugged into the video output. Once you see the “Hello, world!” banner line and the display of the CLKFREQ and CNT registers, you can use your PS2 keyboard to type characters that should be echoed to the TV.

## filetest

To build the filetest test program, cd to the tests\filetest directory and type make. This should run the ZPU C compiler to compile and link the filetest test program. You will find the resulting program in tests\filetest\bin\filetest.zbn. Files with the extension .zbn are ZOG binary images that are ready to load into your Propeller board. To load the filetest test program, type make run. This should load the filetest test program into your board and start it running. To run the filetest program you will need to insert a formatted SD card into your board. The filetest program just writes two files, test.dat and test2.dat to the SD card and reads them back to verify that they were written correctly. It currently only works on the Propeller C3 board.

## xbasic

To build the xbasic test program, cd to the tests\xbasic directory and type make. This should run the ZPU C compiler to compile and link the xbasic test program. You will find the resulting program in tests\xbasic\bin\xbasic.zbn. Files with the extension .zbn are ZOG binary images that are ready to load into your Propeller board. To load the xbasic test program, type make run. This should load the xbasic test program into your board and start it running. The xbasic program is a simple line-oriented Basic interpreter that uses the serial port for input/output. You can try it by typing this simple program:

10 for x=1 to 10

20 printf(“%d %d\n”, x, x*x)

30 next x

You can use the list command to list the program and the run command to run the program. When you run it you will see it print the numbers for 1 to 10 and their squares.

# Building Programs

## Board Configurations

- c3 – this should be used with the Parallax Propeller C3 board when you want to compile code that loads into the 1mb SPI flash. This allows up to 1mb of code and 64k of data using both the SPI flash and SPI SRAM chips on the C3.

- c3ram – this should be used with the Parallax Propeller C3 board when you want to compile and run code that loads into the SPI SRAM chips. This allows up to 64k of code and data and leaves the SPI flash chip alone for other uses.

- sdram – this should be used with the Gadget Gangster Propeller Platform board with the SDRAM module.

## Make Files

common.mk

The test programs are built using the make utility and the include\common.mk makefile fragments. You should be able to look at the makefiles in the tests/hello and tests/filetest directories for an example of how to write your own makefiles to build simple C programs for ZOG. For example, here is the makefile for hello:

TARGET=hello

SOURCES=hello.c initio.c

include $(ZOGPATH)/include/common.mk

The TARGET variable specifies the name of the program to build. The resulting ZOG compile image will have this name with .zbn appended.

The SOURCES variable is a list of source files separated by spaces. If you have more source files than will fit on a line, you can continue them on multiple lines by ending each line with a space followed by a backslash. There should be no space or any other characters after the backslash. For example:

SOURCES=hello.c \

source2.c \

source3.c \

initio.c

The last line should not end in a backslash.

The final line of the hello makefile includes the common.mk makefile fragments that define how ZOG programs should be built.

For a more complex example with multiple source files look at tests/xbasic/makefile.

You can also invoke the ZPU tools directly without using make. Look at common.mk to see which options are needed for the ZPU tools and be sure to include the correct linker script for the board configuration you are targeting. The linker scripts have the same names as the board configurations with .ld appended.

# ZOGLOAD

## Description

ZOGLOAD is a program that runs on the PC under either Windows or Linux and provides a way to download files to any of the supported boards. It can write compiled C programs (in the form of .zbn files) either to memory or an SD card. Programs compiled for the c3 board configuration can be downloaded to flash. Programs compiled for the c3ram or sdram board configurations can be downloaded into RAM.

## Selecting the Port (-p)

You select which communications port is used to talk to your Parallax C3 board using the -p option. You should follow the -p with the communications port identifier. On Windows this can be either COMn or just n. If you don’t specify a port on the zogload command line the default is taken from the PORT environment variable. This is usually setup by the setenv.bat file described in the section on Installing ZOG.

Example

zogload -p COM3

or

zogload -p3

## Selecting the Board Configuration (-b)

You select your board configuration by using the -b option. The two choices for board configuration are c3 for programs intended to run from SPI flash on the C3 and c3ram for programs intended to run from SPI SRAM. The sdram board configuration is for the Propeller Platform board with SDRAM module. If you don’t specify a board configuration on the zogload command line the default is taken from the BOARD environment variable. This is usually setup by the setenv.bat file described in the section on Installing ZOG.

Example

zogload -b c3

or

zogload -b c3ram

or

zogload –b sdram

## Writing a File to Flash or RAM (-wm)

You write a program into memory by using the -wm option. This option should be followed by the name of the compiled ZOG program to write. Programs compiled using the c3 board configuration will be loaded into flash, programs compiled with the c3ram or sdram board configurations will be loaded into RAM. The -wm option is the default if no option is specified before a filename so it can be left out if you want to load a program into memory.

Example

zogload –wm hello.zbn

or

zogload hello.zbn

## Writing a File to the SD Card (-wf)

Example

zogload -wf hello.zbn

## Load the helper program to Propeller RAM (-h)

The –h option is used if the helper bootloader has not been loaded into EEPROM. When –h is specified before a download option like –wf or –wm, the helper program is downloaded to Propeller hub RAM before the ZOG image is downloaded.

Example

zogload –h -wm hello.zbn -r

## Setting the Program Name (-n)

The –n option is used to specify a name to be used by a subsequent option. It is usually used with the –wb to write the zogload.run file or with the –r option to run a program from the SD card.

Example

zogload -n hello.zbn -r

## Writing a Bootloader to the EEPROM (-we)

There are three bootloaders that zogload can write to the EEPROM.

The helper bootloader waits for a second after reset for a packet from the PC to initiate a download to either Propeller memory or the SD flash card. If no packet is received, it mounts the SD card and looks for a file named zogload.run in the root directory. It reads that file and uses its contents as the name of the file to load. For example, if zogload.run contains the string “hello.zbn” that file will be loaded and started. The helper bootloader should be programmed into EEPROM on boards that will be used for ZOG development since it avoids having to download the helper program every time you want to download a ZOG image.

Example

zogload -we helper

The sd bootloader works the same as the helper bootloader except that it doesn’t wait for a packet from the PC but instead immediately starts the program whose name is found in the SD card file zogload.run.

Example

zogload -we sd

The other bootloader is flash. It loads and starts whatever program is programmed in the flash memory. This is the last program written to flash using the –wm command with a program compiled for a flash board configuration, currently only c3.

Example

zogload -we flash

## Setting the File to be Loaded by the Bootloader (-wb)

If you’ve programmed the sd bootloader into EEPROM you will need to write the zogload.run file to the SD card so the bootloader knows which ZOG program to run on reset. You do this with the –wb option. The –wb option writes the name of the most recently mentioned file to zogload.run on the SD card.

Example

zogload –wf hello.zbn –wb

or

zogload –n hello.zbn -wb

## Running a Program (-r)

The –r option runs a previously loaded program. On a board that supports flash memory, it can run the program that is in the flash using just the –r option itself.

Example

zogload –r

In all other cases, there must be a file to run specified on the command line. This can be done in several ways. The file could have just been loaded into memory with the –wm option.

Example

zogload -wm hello.zbn –r

Or, the file could have just been written to the SD card with the –wf option.

Example

zogload –wf hello.zbn –r

Or lastly, the file could already be on the SD card. In that case the –n option can be used.

Example

zogload –n hello.zbn –r

## Terminal Emulation (-t)

Any of the commands can be followed by –t to enter terminal mode after the load is complete. This is a simple terminal emulator that sends the characters typed on the PC keyboard to the Propeller’s serial port on pins 30/31 and displays characters transmitted by the Propeller on the terminal.

Example

zogload -wm hello.zbn -r -t

# C Runtime Library

## Initialization

Many C programs interact with the user by reading text from stdin and writing output to stdout and stderr. The ZOG runtime library (based on newlib) supports this simple terminal I//O in a number of different ways. By default, because embedded programs often don’t have a text interface, the stdin/stdout/stderr files are connected to /dev/null. In other words, reads from stdin will always return EOF and writes to stdout or stderr will just be ignored.

However, it is sometimes useful to be able to use terminal I/O, especially when porting generic C programs to the Propeller. You can arrange to use terminal I/O by including in your program the function _initIO. This function is called by the C startup code before your main function is called. This allows you to setup terminal I/O. One advantage of using _initIO is that you don’t have to modify generic C code to make it work on the Propeller. You just have to add an additional module containing your definition of the _initIO function and link it with the generic C source files.

Here is an example of a simple _initIO function that sets up serial terminal I/O:

#include "propeller.h"

void _initIO(void)

{

InitSerialTerm(

conRxPin,

conTxPin,

conMode,

conBaud,

TERM_IO);

}

The conRxPin, conTxPin, conMode, and conBaud values are defined in the include/board_xxx.h file associated with the board configuration you have chosen. For instance, for the c3 board configuration, they are in the file include/board_c3.h. The TERM_IO parameter says that both input and output will be directed to the serial port.

Here is another example to setup TV output and PS2 keyboard input

#include "propeller.h"

void _initIO(void)

{

InitKeyboardTerm(keybdPinD, keybdPinC);

InitTvTerm(tvPin);

}

Again, keybdPinD, keybdPinC, and tvPin are defined in the relevant board_xxx.h file.

## Serial I/O

int InitSerialTerm(

int rxpin,

int txpin,

int mode,

int baudrate,

int flags);

You can use this function to setup terminal I/O on a serial port. You can setup either input or output or both. It loads the FdSerial driver into a COG and directs stdin, stdout, and stderr to the serial port. The rxpin, txpin, mode, and baud parameters are the same as the corresponding parameters to FdSerial_start.

Example

InitSerialTerm(31, 30, 0, 115200, TERM_IO);

Use TERM_IO to setup both serial input and serial output for stdin, stdout, and stderr. Use TERM_IN to setup just serial input from stdin and TERM_OUT to setup just serial output to stdout and stderr.

## Keyboard Input

int InitKeyboardTerm(

int dpin,

int cpin);

You can use this function to setup terminal input from a PS2 keyboard for stdin. It loads the Keyboard driver into a COG and directs stdin to the keyboard. The dpin and cpin parameters are the same as the corresponding parameters to keybd_start.

Example

InitKeyboardTerm(26, 27);

## TV Output

int InitTvTerm(

int tvpin);

int InitTv2Term(

int tvpin);

You can use these functions to setup terminal output to the TV for stdout and stderr. The InitTvTerm function uses the Parallax TV driver that supports 42 columns by 14 rows. The InitTv2Term function uses the TV_Half_Height driver that supports 44 columns by 54 rows. The tvpin parameter is the same as the corresponding parameter to tvText_start and tv2Text_start.

Example

InitTvTerm(12);

## File I/O

int InitC3FileIO(

int retries);

You can use this function to enable SD card file I/O when using the c3 or c3ram board configurations. The InitC3FileIO function mounts the SD card and redirects all file I/O to the SD card. The retries parameter determines how many times to attempt to mount the SD card. Specify zero for retries to try indefinitely. The C3 file I/O driver uses the same COG that is used by ZOG to manage the SPI flash and SRAM memories. No additional COG is loaded by this function.

Example

InitC3FileIO(10);

# Drivers

The following drivers have been ported to ZOG. Check the header files for them in the include directory for more information on the functions that are available.

## FdSerial

#include “FdSerial.h”

### FdSerial_start

int FdSerial_start(

FdSerial_t *data,

int rxpin,

int txpin,

int mode,

int baudrate);

Initializes and starts a serial driver in a COG.

### FdSerial_stop

void FdSerial_stop(

FdSerial_t *data);

Stops a serial driver.

### FdSerial_rxflush

void FdSerial_rxflush(

FdSerial_t *data);

Empties the receive queue.

### FdSerial_rxcheck

int FdSerial_rxcheck(

FdSerial_t *data);

Gets a byte from the receive queue if one is available. Otherwise, it returns -1.

### FdSerial_rxtime

int FdSerial_rxtime(

FdSerial_t *data,

int ms);

Gets a byte from the receive queue if one is available before the specified timeout in milliseconds. Otherwise, it returns -1 to indicate timeout.

### FdSerial_rx

int FdSerial_rx(

FdSerial_t *data);

Waits for a byte from the receive queue.

### FdSerial_tx

int FdSerial_tx(

FdSerial_t *data,

int txbyte);

Puts a byte into the transmit queue.

## TvText

The following functions are provided in the TvText driver. For more information, see include/TvText.h.

#include “TvText.h”

int tvText_start(int basepin);

void tvText_stop(void);

void tvText_str(char* sptr);

void tvText_dec(int value);

void tvText_hex(int value, int digits);

void tvText_bin(int value, int digits);

void tvText_out(int c);

void tvText_setColorPalette(char* palette);

uint16_t tvText_getTile(int x, int y);

void tvText_setTile(int x, int y, uint16_t tile);

uint16_t tvText_getTileColor(int x, int y);

void tvText_setTileColor(int x, int y, uint16_t color);

void tvText_setCurPosition(int x, int y);

void tvText_setCoordPosition(int x, int y);

void tvText_setXY(int x, int y);

void tvText_setX(int value);

void tvText_setY(int value);

int tvText_getX(void);

int tvText_getY(void);

void tvText_setColors(int value);

int tvText_getColors(void);

int tvText_getColumns(void);

int tvText_getRows(void);

## Tv2Text

The following functions are provided in the Tv2Text driver. For more information, see include/Tv2Text.h.

#include “Tv2Text.h”

int tv2Text_start(int basepin);

void tv2Text_stop(void);

void tv2Text_str(char* sptr);

void tv2Text_dec(int value);

void tv2Text_hex(int value, int digits);

void tv2Text_bin(int value, int digits);

void tv2Text_out(int c);

void tv2Text_setColorPalette(char* palette);

uint16_t tv2Text_getTile(int x, int y);

void tv2Text_setTile(int x, int y, uint16_t tile);

uint16_t tv2Text_getTileColor(int x, int y);

void tv2Text_setTileColor(int x, int y, uint16_t color);

void tv2Text_setCurPosition(int x, int y);

void tv2Text_setCoordPosition(int x, int y);

void tv2Text_setXY(int x, int y);

void tv2Text_setX(int value);

void tv2Text_setY(int value);

void tv2Text_setYhalf(int value);

int tv2Text_getX(void);

int tv2Text_getY(void);

void tv2Text_setColors(int value);

int tv2Text_getColors(void);

int tv2Text_getColumns(void);

int tv2Text_getRows(void);

void tv2Text_button(

int left,

int top,

int width,

int height,

int color,

char* text);

void tv2Text_box(

int left,

int top,

int width,

int height,

int color);

void tv2Text_rectangle(

int left,

int top,

int width,

int height,

int color,

int fill);

## Keyboard

#include “Keyboard.h”

### keybd_start

int keybd_start(

int dpin,

int cpin);

Initializes and starts the keyboard driver in a COG.

### keybd_startx

int keybd_startx(

int dpin,

int cpin,

int locks,

int autorep);

Like start, but allows you to specify lock settings and auto-repeat.

### keybd_stop

void keybd_stop(void);

Stops the keyboard driver.

### keybd_present

int keybd_present(void);

Checks to see if a key is present.

### keybd_key

int keybd_key(void);

Gets a key from the buffer and returns zero if the buffer is empty.

### keybd_getkey

int keybd_getkey(void);

Gets a key and waits if none is currently available.

### keybd_newkey

int keybd_newkey(void);

Clears the buffer and waits for a new key to arrive.

### keybd_gotkey

int keybd_gotkey(void);

Checks to see if there are any keys in the buffer. (How is this different from keybd_present?)

### keybd_clearkeys

void keybd_clearkeys(void);

Clears the key buffer.

### keybd_keystate

int keybd_keystate(

int key);

Get the state of a key.

# Propeller Interface

## Hub Memory Variables

Sometimes it is necessary to place variables in hub memory so that they can be accessed by another COG. You can do this by using the special HUB memory attribute that is defined in propeller.h.

Example

#include “propeller.h”

HUB uint32_t params[2];

This will place the params variable in hub RAM.

## Memory Map

### C3 Memory Map

0x00000000 - 0x0000ffff	Cached external RAM

0x00100000 - 0x001fffff	Cached external ROM

0x10000000 - 0x10007fff  Hub RAM

0x10008000 – 0x1000ffff  Hub ROM

0x18000000 – 0x100081ff  COG memory

### Propeller Platform SDRAM Memory Map

0x00000000 - 0x0fffffff	Cached external memory

0x10000000 - 0x10007fff  Hub RAM

0x10008000 – 0x1000ffff  Hub ROM

0x18000000 – 0x100081ff  COG memory

## Propeller Registers

There are a number of the Propeller registers that are available from C code.

### Hub Variables

CLKFREQ

CLKFREQ isn’t a register. It is a location in hub memory that is initialized with the current clock frequency.

### COG Registers

The following registers are available from C code. Keep in mind that these are the registers of the COG that is running the ZOG virtual machine. For a description of each register see the Propeller Manual.

PAR

CNT

INA

INB

OUTA

OUTB

DIRA

DIRB

FRQA

FRQB

PHSA

PHSB

VCFG

VSCL

## Propeller Functions

### cogid

int cogid(void);

Return the ID of the current COG.

### cognew

int cognew(

int *code,

int size,

int *par);

Allocate a COG, load code into it and start it running passing it par as a parameter. The size parameter gives the size of the code in longs. If par is a pointer it must point to hub memory. The code need not be in hub memory.

### coginit

int coginit(

int cog,

int *code,

int size,

int *par);

Load code into cog and start it running passing it par as a parameter. The size parameter gives the size of the code in longs. If par is a pointer it must point to hub memory. The code need not be in hub memory.

### cogstop

void cogstop(

int cog);

Stop cog from running and make it available again for allocation by cognew.