“XGS BASIC”
A Quick Start Guide

# Overview

The XGS BASIC was developed by David Betz as a mixed C / BASIC syntax compiler and runtime virtual machine. At its current state (as of V1.4) the XGS BASIC supports computation and NTSC video display output utilizing either bitmap or tile graphics. The XGS BASIC is also capable of reading in gamepad button states as well as generating sounds and more. Since the XGS BASIC is being continually developed and is completely open sourced many new capabilities will be added. Some of the defining features are listed below.

- Works on both XGS PIC 16-Bit and XGS AVR 8-Bit (with some limitations on the AVR)

- Supports Tile Map and Bitmap Grahpics (XGS PIC only).

- Operates the XGS AVR out of chip flash and can operate the XGS PIC out of chip flash, RAM, or SD card.

- Programs over serial cable (no programmer required other than initial Virtual Machine flashing)

- Standard operations such as RND(), ABS(), and TEXT() plotting functions.

| TIP | Make sure to read the XGS BASIC user manual for the latest available features in the current operating version. This document is meant as a quick start guide and introduction to the XGS BASIC. |
| --- | --- |

The XGS BASIC has three main components to it: the XGSBASIC code written by a user, the XGS BASIC compiler and program downloader, and finally the Virtual Machine that runs the intermediate code. The interaction between these systems can be seen in Figure 1 below.

Figure 1 – XGS BASIC overview.

Referring to Figure 1, the user writes their BASIC source code in a normal text editor such as notepad, TextPad, etc. After saving the file the XGS BASIC compiler checks for any syntax errors and generates the intermediate code (I-code) and optionally downloads the I-code to the XGS via a serial connection. If the code was downloaded to the XGS device successfully the Virtual Machine located on the XGS begins running the BASIC byte code. Also, while the BASIC code is operating on the XGS the serial port continues communicating with the BASIC compiler/downloader and can provide user debug output as well as receive new byte code programs.

# Installation

XGS BASIC Compiler

There are two parts to the XGS BASIC that need to be installed. The first is the XGS BASIC compiler executable (xgsbasic.exe). There is no complex installation procedure for the compiler, simply copy the program from the XGS DVD-ROM or download and unzip the software onto the local hard drive.  Since this is a command line tool to be run in DOS (possibly future versions in Linux, MAC OS) it is easiest to put it into a directory right off the root drive so you do not have to navigate too far to find it.

XGS BASIC Virtual Machine

The XGS BASIC Virtual Machine (VM) resides on the XGS PIC/AVR and is therefore created using MPLAB or AVR Studio. If you choose so you can compile the VM yourself or you can use the pre-compiled HEX files (.hex) and simply load the VM into flash using the PICKit 2 or the AVR ISP programmers. For help on loading the pre-compiled hex files onto your specific development board, see the XGS PIC/AVR User’s Manual.

| NOTE | Complete source code is given for both the compiler and the virtual machine. Thus you can either load the source code into a compiler such as Visual Studio for the XGS BASIC compiler and generate an executable, or MPLAB / AVR Studio for the Virtual Machine. Nevertheless we have already precompiled all required files to save most of the hastle. |
| --- | --- |

# Usage

Typical operation will follow these steps:

- Write or modify your BASIC source code using an editor. When finished editing, save the file before continuing.

- Use the XGS BASIC Compiler to create and download the I-Code to the XGS development systems.

- Verify desired functionality and repeate steps 1-2.

We will now consider an example usage such as changing the text output in the “Game of Life”, cellular automaton demo. Let’s pretend that you installed (copied) the latest XGS BASIC package onto your hard drive at location C:\XGSBASIC. If you copied the entire source contents there will be some example BASIC files such as life.bas in this folder. Open the file life.bas in a text editor like notepad.

Scroll down towards the end of the file and find the code shown below.

print "clearing screen"

call clear(3)

call text(10, 5, "The Game of Life")

call fillrect(leftmargin - 1, topmargin - 1, worldw * (cellw + 1) + 1, worldh * (cellh + 1) + 1, 0)

This code set calls the built in function “print” that is a debug statement that will send the given text back through the serial port to the terminal program. Next the clear subroutine is called which clears the screen with the given color code passed in. After the clear call the text subroutine is executed that prints the text onto the screen with X coordinate 10, Y coordinate 5. Finally a filled rectangle call is drawn. Further information on the available instructions and XGS BASIC usage can be seen in the xgsBasicManual.pdf. Edit this code section and replace the text “The Game of Life” with “The XGS Rules”. Now save the file.

Next we are going to compile the edited life.bas into intermediate code. Open up a DOS console window by going to the Microsoft Windows Start ->Run menu. Type cmd.exe as shown in Figure 2 below.

Figure 2 – Starting a DOS terminal window.

Now press the OK button in the Run window and a DOS terminal window will open up as can be seen in Figure 3. Now navigate to the location where you installed the XGS BASIC compiler and code. If the installation folder was C:\XGSBASIC you would type:

cd C:\XGSBASIC

Figure 3 – Navigating to the XGS BASIC folder.

The next step is to prepare the hardware for download. If you haven’t done so already, turn on the XGS development board and program the virtual machine into the flash. If using the XGS PIC use the xgspicvm.hex. If using the XGS AVR program with the xgsavrvm.hex. Now connect a serial port cable to the XGS development board from the computer running the XGS BASIC compiler. If you need help in determing how to connect to the serial port on the XGS system look at the XGS PIC/AVR Users Manual under the UART section for pictures and description. Also you will need to know what serial port number on your computer is connected to the XGS PIC/AVR.

At this point your XGS PIC/AVR board should be powered ON with the XGS BASIC virtual machine running on the sytem. Also a serial cable or USB to serial converter is connecting the XGS board to your computer.  Now invoke the XGS BASIC compiler and serial downloader by issuing the following command.

xgsbasic.exe –p COM6 life.bas

Where COM6 in this example refers to the computers serial port number 6. Your port number may be different. If there was no problems in compiling and downloading, the terminal window will resemble Figure 4 below.

Figure 4 – Compiled and Download BASIC program.

The XGS development kit should now be displaying the “Life” demo on the NTSC television screen with your modified text string at the top. Kill the xgsbasic.exe program by pressing “Ctrl+C”. Now you are free to edit the life.bas file again (remember to save) and re-compile and download to the XGS.

There are a few different command line options for the XGS BASIC compiler. Table 1 shows the options available.

Table 1 – XGS BASIC compiler command line options.

Option Description

-c Compile only and do NOT download.

-o Change the output i-code file name. Default is input filename with .bai extension.

-p COMx Serial communications port to use where x is a number.

-r (XGS PIC only) Download and execute out of SRAM instead of flash (saves flash write cycles)

-d Displays the compiled I-code and symbol tables

# Summary

The XGS BASIC allows developers to quickly write and run software programs that take advantage of the available hardware features of the XGS development kits. In addition to the easy human readable high-level language coding, the XGS BASIC is very simple to operate in contrast to a full blown C compiler where the user must setup projects, linkers, programmers, compilers in order to achieve their desired output.

There are a few other reference materials to look at in addition to this quick start guide. These are listed below.

- xgsBasicManual.pdf – contains the XGS BASIC language description and a list of buil-in functions.

- README.TXT – contains additional usage instructions on the XGS BASIC compiler.

- www.xgamestation.com/phpbb/ - User community forums where other XGS BASIC users communicate and get help.